Alamance County

Residential Plan Review Requirements

The following is provided as a guide for obtaining Building plan approval for residential single-family dwellings, modular homes, and additions and renovations in Alamance County.

It applies to all plans submitted under the 2018 NC Residential code.

- General

  - All plans submitted for plan review must be submitted to planreview@alamance-nc.com

  - Plans must be submitted as one file in PDF format

  - Floor plan layout with all rooms / areas labeled for use

  - Window and door sizes

  - Site plan

- Insulation

  - Insulation values for floors, walls and ceilings. Provide detail for monolithic slab foundations

- Structural

  - Footing and foundation details

  - Thickened slab details for poured concrete foundations

  - Complete details for floor joists and girders for all floor systems

  - Sealed floor truss plans if they are used

  - Wall construction details, including stud height, size, spacing and wall bracing method / details

  - We are no longer requiring roof truss drawings at plan review time, these should be placed in the permit box for the framing inspection. A layout is required as well

  - If roof is stick framed, complete roof structural including - size, spacing, direction of all members including ceiling joists and all rafters

  - All header and girder sizes with bearing and attachment details and locations (For LVL headers, the plans should just state ”LVL header”)

  - The cut sheets for each LVL header must be in the permit box at the time of framing inspection.

  - Point loaded LVL headers- cut sheet must be sealed by engineer

  - Any steel girders must be sized and sealed by engineer

- Decks

  - Dimensional plans including:

  - Footings

  - Post locations

  - Girder and joist size, span, spacing and direction

- Fire protection - duplex, townhome, multi-family

  - Separation wall design and details

  - Fire separation details for soffit